Lecture 2 - ANSWER KEY

Part 1: Concept Check

1. bandwidth

2. propagation

3. transmission

4. bandwidth; propagation delay

5. transient

6. persistent

7. in flight (or: on the link)

Part 2: Calculation Problems

Problem 1

a) Transmission delay = 1000 bytes × 8 bits/byte / 10,000,000 bps = 8000/10,000,000 = 0.0008 s = 0.8 ms

b) Total delay = 0.8 ms + 5 ms = 5.8 ms

c) BDP = 10,000,000 bps × 0.005 s = 50,000 bits

Problem 2

a) 100-byte = 800 bits

Link 1: 800/100,000,000 + 0.05 = 0.000008 + 0.05 = 50.008 ms

Link 2: 800/10,000,000 + 0.005 = 0.00008 + 0.005 = 5.08 ms

Link 2 is faster for small packets

b) 1 MB = 8,000,000 bits

Link 1: 8,000,000/100,000,000 + 0.05 = 0.08 + 0.05 = 130 ms

Link 2: 8,000,000/10,000,000 + 0.005 = 0.8 + 0.005 = 805 ms

Link 1 is faster for large packets

c) For small packets, propagation delay dominates; for large packets, transmission delay dominates.

Problem 3

a) BDP(capacity) = 1,000,000 bps × 0.25 s = 250,000 bits

b) Transmission = 10,000/1,000,000 = 0.01 s = 10 ms

Total = 10 ms + 250 ms = 260 ms

c) Round trip = 2 × (transmission + propagation) = 2 × 260 ms = 520 ms

Part 3: Pipe Diagrams

Problem 4

b) BDP (capacity)= 4 bps × 3 s = 12 bits

c) Transmission delay = 8 bits / 4 bps = 2 seconds

d) The packet width in the diagram = 2 seconds (transmission delay)

Problem 5

Link B is taller (1000 bps > 100 bps)

Link A is wider (10 s > 1 s)

Both have the same area (BDP = 1000 bits)

Part 4: Timing Diagrams

Problem 6

TD = 4/2 = 2s, to deliver 1 bit: TD = ½ = 0.5s

Total delay = TD + PD = 2 + 1 = 3s, for each bit: 0.5 + 1 = 1.5s

Bit 1: sent t=0, arrives t=1.5

Bit 2: sent t=0.5, arrives t=2

Bit 3: sent t=1.0, arrives t=2.5

Bit 4: sent t=1.5, arrives t=3

Part 5: Queuing and Overload

Problem 7

a) Each packet: 8000 bits / 1,000,000 bps = 0.008 s = 8 ms

b) A finishes at t=8ms, B starts at t=8ms

c) B queuing delay = 8 ms, C queuing delay = 16 ms

d) Transient - packets arrived simultaneously, but after queue drains, no more overload

Problem 8

a) 16 Mbps

b) 10 Mbps

c) Persistent - incoming rate exceeds outgoing capacity continuously

d) Queue will fill up, packets will be dropped

e) Solutions: upgrade link, tell senders to slow down (congestion control)

Part 6: Challenge Problem

Problem 9

a) Fiber: 8,000,000,000 bits / 1,000,000,000 bps + 0.01 s = 8 s + 0.01 s = 8.01 seconds

b) USB drive: 30 minutes = 1800 seconds (loading time negligible)

c) Fiber is much faster (8 seconds vs 30 minutes)

d) Break even: 1,000,000,000 bps × 1800 s = 1,800,000,000,000 bits = 225 GB

For files larger than ~225 GB, driving is faster!

e) Physical transport has huge "bandwidth" (can carry many drives), but high latency.

For very large data transfers, the bandwidth advantage overcomes the latency penalty.