# Lecture 3 Worksheet

Routing Principles

CMSI 3550 | Networks and Internets

## Part 1: Core Concepts

1. Briefly explain why routing is needed in networks. What problem does it solve compared to simpler approaches like full-mesh or single-link topologies?

2. What are the three main challenges that make routing a hard problem?

(a) Challenge 1:

(b) Challenge 2:

(c) Challenge 3:

3. Distinguish between intra-domain and inter-domain routing protocols. Give one example of each type.

## Part 2: Destination-Based Forwarding

Consider the following network topology:

A --- R1 --- R2 --- B ¦ ¦ R3 --- R4 --- C ¦ D

4. Fill in R1's forwarding table so packets can reach all destinations:

R1's Forwarding Table:

| Destination | Next Hop |
| --- | --- |
| A |  |
| B |  |
| C |  |
| D |  |

5. Explain the difference between routing and forwarding. Which one is "local" and which one is "global"?

## Part 3: Routing State Validity

6. What two conditions must be satisfied for a global routing state to be valid?

(a) Condition 1:

(b) Condition 2:

7. Explain why, in destination-based forwarding, a packet stuck in a loop can never escape.

8. Consider this network with the following forwarding tables:

R1 -------- R2 ¦ ¦ ¦ ¦ A - R3----------R4 - B

Given forwarding tables (for destination B):

R1: next-hop = R2    R2: next-hop = R4    R3: next-hop = R1    R4: next-hop = B

(a) Draw the directed delivery tree for destination B.

(b) Is this routing state valid for destination B? Why or why not?

## Part 4: Least-Cost Routing

Consider the following weighted network topology:

1 3 2 A ----- R1 ----- R2 ----- B ¦2 ¦1 R3 ----- R4 ----- C 4 1

9. What are two properties we assume about link costs? Why do these assumptions simplify routing protocols?

10. Using the weighted topology above:

(a) What is the least-cost path from A to B? What is its total cost?

(b) What is the least-cost path from A to C? What is its total cost?

(c) Why does least-cost routing guarantee no loops?

## Part 5: Static and Direct Routes

11. What is the difference between a direct/connected route and a static route?

12. Give one advantage and one disadvantage of using static routes instead of a dynamic routing protocol.

## Bonus Challenge

13. If a network has N routers and each router needs to reach M destinations, what is the minimum number of forwarding table entries across all routers? Under what conditions would this minimum be achieved?