# Intro Lecture - Answer Key

## Part 1: Internet Design Challenges

1.1a: (0.99)^50 = 0.605 or 60.5%

1.1b: 1 - 0.605 = 0.395 or 39.5%

1.1c: (0.98)^10 = 0.817 or 81.7%

1.1d: n ≤ ln(0.999)/ln(0.995) ≈ 0.2 hops (essentially must be direct!)

1.2a: 4,125,000 m; 300,000,000 m/s; 13.75 ms

1.2b: (2/3)13.75 = 9.1 ms

1.2c: 3×10⁹ × 0.01375 = 41,250,000 cycles; 82,500,000 round-trip

1.2d: 2 × 8300 / (200,000 km/s) = 83 ms

## Part 2: Network Layers

2.1: Router forwards (L2, L3); Browser displays (L7); Voltages (L1); Retransmit (L4); WiFi connect (L1, L2); DNS (L7); Berkeley→Tokyo (L3)

2.2: Application, Transport, Internet, Link, Physical. Router doesn't need L4/L7 because it only forwards packets (doesn't create/consume application data or manage connections).

## Part 3: Headers and Encapsulation

3.1c: L2 header changed (new source/dest for each link); L3 and L4 stay same

3.2: L2=A, L3=B, L4=C

3.3a: Reads L2 ✓, Reads L3 ✓, Removes L2 ✓, Adds new L2 ✓

3.3b: L2: R1→R2, L3: A→B, L4: 5000→80, "Hello"

## Part 4: Best-Effort Service

4.1a: (0.98)^5 = 0.904 or 90.4%

4.1b: 1000 × 0.096 = 96 packets

4.1c: 1-(1-0.904)^n ≥ 0.99 → n ≥ 2

4.2a: Layer 4: Transport

4.2b: Not all applications need reliability; video streaming prefers low latency over perfect delivery

4.2c: Video streaming, voice calls, online gaming

## Part 5: Putting It All Together

5.2a: 3 times (WiFi→Ethernet, Ethernet→Optical, Optical→Ethernet)

5.2b: No—L3 header always contains end-to-end addresses

5.2c: Layering / Abstraction

## Part 6: Challenge Problems

6.1a: (0.9)^3 = 0.729

6.1b: 1/0.729 ≈ 1.37 attempts

6.1c: 1.37 × 50 = 68.5 ms

6.2a: 1000 + 18 + 20 + 20 = 1058 bytes

6.2b: 58/1058 = 5.5%

6.2c: 58/158 = 36.7%

6.2d: 58/0.10 - 58 = 522 bytes

6.3a: (0.8)^10 = 0.107 or 10.7%

6.3b: (0.8)^1000 ≈ 0 (essentially impossible)

6.3c: Layering—can change one layer without affecting others; also backward compatibility