Lecture 2 Worksheet

Links: Bandwidth, Delay, and Pipe Diagrams

CMSI 3550 | Networks and Internets

Key Formulas Reference Packet Delay = Transmission Delay + Propagation Delay + Queuing Delay Transmission Delay = Packet Size / Bandwidth (Time to push all bits onto the link) Propagation Delay = Distance / Speed of light in medium (Time for a bit to travel across the link) Bandwidth-Delay Product (capacity) = Bandwidth × Propagation Delay (How many bits can be "in flight" on the link at once) Unit conversions: 1 Mbps = 1,000,000 bits/second 1 ms = 0.001 seconds 1 byte = 8 bits

Part 1: Concept Check

Fill in the blanks with the correct term.

1. The _____________ of a link describes how many bits can be sent per unit time.

2. The _____________ delay is the time it takes for a single bit to travel from sender to receiver.

3. The _____________ delay is the time it takes to push all bits of a packet onto the link.

4. In a pipe diagram, the height represents ____________ and the width represents _____________.

5. When two packets arrive at a router simultaneously, this is called __________ overload.

6. When a router has more incoming traffic than outgoing capacity over time, this is called _____________ overload.

7. The bandwidth-delay product tells us how many bits can be _____________ at any instant.

Part 2: Calculation Problems

Show your work for full credit.

Problem 1 A link has the following properties: Bandwidth: 10 Mbps (10,000,000 bits/second) Propagation delay: 5 ms (0.005 seconds) a) How long does it take to transmit a 1000-byte packet? Answer: _______________________________ b) What is the total packet delay (from first bit sent to last bit received)? Answer: _______________________________ c) What is the capacity of this link? Answer: _______________________________

Problem 2 Consider two links: Link 1: Bandwidth = 100 Mbps, Propagation Delay = 50 ms Link 2: Bandwidth = 10 Mbps, Propagation Delay = 5 ms a) For a 100-byte packet, which link delivers it faster? Show calculations. Link 1 delay: _______________________________ Link 2 delay: _______________________________ Better link for 100-byte packet: _______________________________ b) For a 1 MB (1,000,000 byte) packet, which link delivers it faster? Link 1 delay: _______________________________ Link 2 delay: _______________________________ Better link for 1 MB packet: _______________________________ c) Explain in one sentence why the answers differ. _________________________________________________________________

Problem 3 A satellite link has: Bandwidth: 1 Mbps Propagation delay: 250 ms (satellite is far away!) a) What is the capacity? Answer: _______________________________ b) If you send a 10,000-bit packet, how long until the last bit arrives? Answer: _______________________________ c) After you send the packet, you wait for an acknowledgment (also 10,000 bits). What is the total round-trip time (send packet + receive ack)? Answer: _______________________________

Part 3: Pipe Diagrams

Problem 4 Consider a link with Bandwidth = 4 bps and Propagation Delay = 3 seconds. a) Draw the pipe diagram showing the link dimensions. Label the height (bandwidth) and width (propagation delay). b) What is the capacity? _______________ c) If you send an 8-bit packet, what is the transmission delay? _______________ d) In the pipe diagram, how wide would this 8-bit packet appear? _______________

Problem 5 Two links have the same capacity of 1000 bits. Link A: Bandwidth = 100 bps, Propagation Delay = 10 seconds Link B: Bandwidth = 1000 bps, Propagation Delay = 1 second Sketch both pipe diagrams. Which pipe is taller? Which is wider? Link A: Link B: Taller pipe: _______________ Wider pipe: _______________

Part 4: Timing Diagrams

Problem 6 Complete the timing diagram for sending a 4-bit packet. Bandwidth: 2 bps (each bit takes 0.5 seconds to transmit) Propagation Delay: 1 second Fill in the times when each bit is sent and received: Sender (A) Receiver (B) ────────── ──────────── t = 0.0s bit 1 sent; t = ____ bit 1 arrives. t = ____ bit 2 sent; t = ____ bit 2 arrives. t = ____ bit 3 sent; t = ____ bit 3 arrives. t = ____ bit 4 sent; t = ____ bit 4 arrives. Total packet delay: _______________________________

Part 5: Queuing and Overload

Problem 7 A router has one outgoing link with bandwidth 1 Mbps. It receives the following packets: t = 0 ms: Packet A arrives (8000 bits) t = 0 ms: Packet B arrives (8000 bits) t = 0 ms: Packet C arrives (8000 bits) a) How long does it take to transmit each packet? Answer: _______________________________ b) The router sends A first. When does A finish transmitting? When does B start? A finishes: _____________ B starts: _____________ c) What is the queuing delay for packet B? For packet C? B queuing delay: _____________ C queuing delay: _____________ d) Is this transient or persistent overload? Why? _________________________________________________________________

Problem 8 A router has an outgoing link with bandwidth 10 Mbps. It receives packets from two incoming links: Incoming Link 1: sends packets at 8 Mbps Incoming Link 2: sends packets at 8 Mbps a) What is the total incoming rate? _______________ b) What is the outgoing rate? _______________ c) Is this transient or persistent overload? _______________ d) What will eventually happen? _______________________________________________ e) Name two solutions to this problem: 1. _______________________________________________ 2. _______________________________________________

Part 6: Challenge Problem

Problem 9 You need to transfer a 1 GB file. You have two options: Option A: Fiber optic link Bandwidth: 1 Gbps (1,000,000,000 bits/second) Propagation delay: 10 ms Option B: Put the data on a USB drive and drive it across town "Bandwidth": 1 GB fits on the drive "Propagation delay": 30 minutes to drive across town a) How long does it take to transfer via the fiber link? (1 GB = 8,000,000,000 bits) Answer: _______________________________ b) How long does it take via the USB drive? Answer: _______________________________ c) Which is faster for this single 1 GB transfer? Answer: _______________________________ d) At what file size does the two options break even? Answer: _______________________________ e) This illustrates a famous quote: "Never underestimate the bandwidth of a station wagon full of tapes hurtling down the highway." Explain why this is sometimes true. _________________________________________________________________ _________________________________________________________________

bandwidth advantage overcomes the latency penalty.